Team members: Mohammed Nafeh

Chase Callies

GitHub: https://github.com/ChaseC130/Final-Project

Team member roles: most of our time was spent troubleshooting issues with the robot itself. Once that was solved we worked to get navigation and then mapping working.

Mohammed: navigation

Chase: mapping

Main Features:

- Detects:

  - solid white lane line

  - orange end-of-road marker

- Uses HSV color thresholding with OpenCV

- Publishes robot velocity commands with /cmd_vel

- Performs:

  - lane centering

  - lane recovery when lines are lost

  - U-turn behavior

  - intersection handling

White Line Tracking:

The robot primarily follows the solid white line on the right side of the lane.

The code smooths lane positions and steering values to reduce jitter and unstable movement.

If lane lines are lost, the robot performs a recovery sweep, rotating until a lane line is found

If the robot detects orange tape at the end of the road:

The robot performs:

- rotate left

- forward creep behavior

- differentiates between lane and reflection/glare by looking for blobs of light

- lane reacquisition after the turn (not 100% working)

- Debugging filter for camera to see what the robot sees

Main Nodes:

Camera_driver_node

Purpose:

Reads camera devices

Publishes ROS2 image topics

Publishes:

/camera_1/image_raw

/camera_2/image_raw

Edubot_autonomy_node

Purpose:

Detects lane markings

Computes steering corrections

Controls robot movement

Subscribes:

/camera_1/image_raw

/camera_2/image_raw

Publishes:

/cmd_vel

Lidar_mapper_node (not working)

Purpose:

Builds occupancy grid maps

Uses LiDAR and odometry data

Saves maps to disk

Subscribes:

/scan

/wheel/odometry

Publishes:

/map

Outputs: (not working)

.yaml map file

.pgm occupancy image

test_sensor_publisher

Purpose:

Simulates sensors for testing without hardware (debugging)

Publishes:

/scan

/wheel/odometry

Camera image topics

Architecture Diagram and Flowchart: